# SCARAB: Design Document

For submission to SETU

Author: Stuart Rossiter

Student Number: C00284845

Course: BSc (Hons.) Software Development

Supervisor: Joseph Kehoe

Contents

1. SCARAB Program	3

1.1	Software Architecture	3

1.1.1	PySide	3

1.1.2	Python	3

1.1.3	pySerial	3

1.1.4	File System	3

1.1.5	External Hardware (SCARAB Device)	4

1.1.6	System Services	4

2. SCARAB Device	5

2.1	Hardware Architecture	5

2.1.1	Arduino MEGA 2560 Rev 3	6

2.1.2	Adafruit 4090 USB C Breakout Board	6

2.1.3	5V to 3.3V Buck Converter	6

2.1.4	Module Connector Interface / SCARAB Connector	6

2.1.5	EEPROM	6

2.1.6	Logic Level Converter	6

2.1.7	Cartridge Port	6

2.1.8 Game Cartridge	6

2.2	Circuit Diagrams	7

2.2.1	SCARAB Device	7

2.2.2	SNES Module	8

2.2.3	N64 Module	9

2.2.4	GBA Module	10

2.2.5	NES Module	11

3. Bibliography	12

# SCARAB Program

## Software Architecture

### PySide

For GUI development, I am using Python’s own binding of the GUI toolkit Qt, developed by The Qt Company themselves. The binding, known as PySide, is cross-platform, much like Qt. The PySide binding is open source under the GPLv3 and Qt commercial license (pyside, 2021).

### Python

The application logic will be handled using Python. Python, created by Guido van Rossum in 1991, is a general-purpose language with a simple syntax, similar to the English language (W3Schools, n.d.). Given its simplicity, it allows for rapid development, aided by its abundance of external libraries.

### pySerial

The pySerial library encapsulates serial port access, providing cross-platform backends for Python (pyserial, 2022). This will be used to interface with the SCARAB Device.

### File System

The SCARAB program will need to store backed up save files. As such, interaction with the PC’s file system is necessary.

### External Hardware (SCARAB Device)

pySerial is being used to communicate with external hardware. In this case, it is communicating with the SCARAB Device itself. Specifically, it is communicating with the Arduino which powers the SCARAB.

### System Services

The SCARAB PC Program will need to make use of some system libraries such as time for any necessary delays while the SCARAB Device connects, and sys for exiting.

# SCARAB Device

## Hardware Architecture

### Arduino MEGA 2560 Rev 3

Based on the ATmega2560 microcontroller, the Arduino Mega 2560 Rev3 contains everything needed to support the ATMega2560, including 54 digital I/O pins, 16 analog inputs, 4 hardware serial ports, a 16MHz crystal oscillator, USB connection, and a power jack (Arduino, 2025). The Arduino serves as the intermediary device which translates between what the PC wants, and what the cartridge expects.

### Adafruit 4090 USB C Breakout Board

The Adafruit USB C board will serve as the main power source for the cartridges and the EEPROM. While a microcontroller has enough power to interface with the cartridge, it doesn’t support the Amps necessary to also power it. Using a USB-C cable, the SCARAB could draw the power necessary from a USB port on the PC. While this will require 2 USB cables to be connected between the SCARAB and the PC, it’s preferable to the alternative of batteries.

### 5V to 3.3V Buck Converter

Not only do some cartridges such as the GBA or N64 use 3.3V logic, but they utilise 3.3V VCC too. The logic level converters would not work for the level of amps required, so a new solution was needed. This is where buck converters are being used. Buck converters convert a DC voltage to a lower DC voltage, such as 5V to 3.3V in this case (Yates, 2024).

### Module Connector Interface / SCARAB Connector

A series of pins and pin headers, the Module Connector and SCARAB Connector serve as a link between the SCARAB Device and its various cartridge port modules.

### EEPROM

The Electronically Eraseable Programmable Read Only Memory (EEPROM) serves as the identification chip for the individual modules. The SCARAB will need to adjust itself based on the type of cartridge it is reading from, so the EEPROM will store the necessary data to assist in this.

### Logic Level Converter

As mentioned above, some cartridges use 3.3V logic, where the Arduino MEGA 2560 uses 5V logic. These logic level converters will translate between the Arduino and the cartridge in these cases. They will not be present on every module, only the ones that require 3.3V.

### Cartridge Port

The cartridge ports serve as an interface between the Arduino and the cartridges themselves. They are fit for purpose, and mimic the port contained within the respective consoles that the cartridges are designed for.

### 2.1.8 Game Cartridge

The game cartridges themselves are the main purpose of the SCARAB. They are being tested and checked for any issues, and having their save data backed up.

## Circuit Diagrams

### SCARAB Device

### SNES Module

### N64 Module

### GBA Module

### NES Module

# Algorithm Pseudocode

## SCARAB

### Identify Module

func identifyModule() {

Initialize EEPROM Connection

value = Read From EEPROM

Close EEPROM Connection

switch (value) {

case “SNES”:

globalFunctionPointers = snesFunctionPointers

case “NES”:

globalFunctionPointers = nesFunctionPointers

case “GBA”:

globalFunctionPointers = gbaFunctionPointers

case “GBC”:

globalFunctionPointers = gbcFunctionPointers

case “N64”:

globalFunctionPointers = n64FunctionPointers

}

}

## SNES

### Read Cartridge

func readSnesCartridge(address) {

Place address on address lines

Set RD line to active low

byte = Read from data lines

return byte

}

### Write Cartridge

func writeSnesCartridge(address, byte) {

Place address on address lines

Place byte on data lines

Set WR line to active low

}

### Dump Save

func dumpSnesSave(saveSize, romLayout) {

Create buffer[1024] to store data

switch (romLayout) {

case “LoRom”:

startLocation = LoRomSaveLocation

case “HiRom”:

startLocation = HiRomSaveLocation

case “ExHiRom”:

startLocation = ExHiRomSaveLocation

}

for address in range startLocation to startLocation + saveSize {

buffer[(address – startLocation % 1024)] = readSnesCartridge(address)

checksum for buffer XOR new byte of data

if buffer is full {

Send buffer to PC

Send checksum to PC

clear buffer

if PC returns OK then continue

else error

}

}

}

### Restore Save

func restoreSnesSave(saveSize, romLayout) {

Create buffer[1024] to store data

switch (romLayout) {

case “LoRom”:

startLocation = LoRomSaveLocation

case “HiRom”:

startLocation = HiRomSaveLocation

case “ExHiRom”:

startLocation = ExHiRomSaveLocation

}

for address in range startLocation to startLocation + saveSize {

if buffer is empty { //if (address – startLocation % 1024) == 0

Request data from PC

Request Checksum from PC

Validate Checksum

if OK then ACK

else ERR

}

writeSnesCartridge(address, buffer[(address – startLocation % 1024)])

}

Send OK to PC

}

### Calculate Checksum

## NES

### Read Cartridge

func readNesCartridge(address, romArea) {

if romArea is “PRG” {

Place address on CPU address lines

set CPU RD to active low

byte = read from CPU data lines

}

else romArea is “CHR” {

Place address on PPU address lines

set PPU RD to active low

byte = read from PPU data lines

}

return byte

}

### Write Cartridge

func writeNesCartridge(address, romArea, byte) {

if romArea is “PRG” {

Place address on CPU address lines

Place byte on CPU data lines

set CPU WR to active low

}

else romArea is “CHR” {

Place address on PPU address lines

Place byte on PPU data lines

set PPU WR to active low

}

}

### Dump Save

func dumpNesSave(saveSize) {

}

### Restore Save

### Calculate Checksum

## GBA

### Read Cartridge

### Write Cartridge

### Dump Save

### Restore Save

### Calculate Checksum

## GB/GBC

### Read Cartridge

### Write Cartridge

### Dump Save

### Restore Save

### Calculate Checksum

## N64

### Read Cartridge

func readN64Cartridge(address) {

Place upper half of address on address lines

Latch upper half of address

Place lower half of address on address lines

Latch lower half of address

Set RD line to active low

byte = Read from data lines

return byte

}

### Write Cartridge

func writeN64Cartridge(address, byte) {

Place upper half of address on address lines

Latch upper half of address

Place lower half of address on address lines

Latch lower half of address

Put data on data lines

Set WR to active low

}

### Dump Save

### Restore Save

### Calculate Checksum

# User Interface

## Main Menu

### SCARAB Found

### SCARAB Not Found

## Modules

### Module Inserted

### 4.2.2 Module Not Inserted

## Check Health

### Cartridge Inserted

### Cartridge Not Inserted

### Health Check In Progress

### Health Check Done

### Health Check Results

# Bibliography

pyside, 2021. GitHub - pyside/pyside-setup: Git super project for PySide. [Online] 
Available at: https://github.com/pyside/pyside-setup
[Accessed 10 February 2026].